Knocking off the Rust: Basics of Blade Maintenance

- Introduction

  - Importance of blade maintenance

  - Overview of common issues and basic care

- Tools and Materials Needed

  - List of essential tools

    - 3M Metal Finishing Pad: Ideal for removing rust and polishing the blade.

    - Max Wax from Darkwood: Provides a protective layer to prevent rust.

    - Small Fine File: For removing blade and hilt nicks.

    - Applicator Cloth: For applying oils and wax.

  - Optional tools for advanced care

    - Gun Oil (especially the wipes): Lubricates and protects the blade.

    - Drywall Sanding Sponges: Effective for initial cleaning and rust removal.

    - Rust Erasing Block: For stubborn rust removal.

    - Scissors: Useful for cutting tapes and cloths.

    - Electrical Tape: Can be used for temporary repairs and grip adjustments.

    - Rubber Mallet: For setting a tips

    - Razor Box Knife: Cutting off old tape and tips.

    - Wood Restorer: Maintains and restores wooden handles.

    - Mink Oil: Conditions and protects leather components.

- Cleaning and Maintenance Techniques:

  - Removing Large Nicks: Use the file, pushing in ONE direction, only slightly crosswise, down the length of the blade. Avoid pulling or scrubbing in one location.

  - Removing Rust:

    - Use the red hand-pad. It's acceptable to push, pull, or scrub with the hand pad to remove rust.

    - CAUTION: The red hand-pad and file can leave scratches on highly polished or blackened surfaces.

  - Final Treatment:

  - Rub the entire article with a light layer of Max Wax (including the hilt, handle, pommel, and blade).

  - Pro-Tip: Do not wash the cloth after use. The more wax in the cloth, the better the coating capability later down the road.

- Inspecting for Damage

  - Common types of damage (nicks, bends, etc.)

  - Checking Tips

- Preventative Care

  - Proper storage

  - Regular maintenance routines

- Q&A Session